ADVANCED AVIATION

# C-172 CHECKLIST

N172WA & N52741

### PREFLIGHT INSPECTION

### CABIN

### Control Lock………………………..………………Remove Hobbs/Tach……………………………..…………….Check

### AROW………….…………………………..……………Check

### Master Switch…….……………………………………….On

Fuel Guages..………………..…..……….………….Check

Lights…………………………..………………………..Check

Flaps………………………..…………………………….Down

Master Switch…………..…………………….…………..Off

#### EMPENNAGE

### Baggage Door………………………………………..Secure

### Horiz.l Stab. & Elevator……..…………………….Check

##### Vert.Stab & Rudder.…………………….………….Check

Tie Down…………………….………………………Remove

Antennas & Lights……………………….…………..Check

#### RIGHT WING

##### Flap, Aileron, Wing Tip………...….…..………….Check

Leading Edge………..………..……….….………….Check

Tie Down………………..…………………………..Remove

Fuel Level………….……………………..…………….Check

Fuel Sump…………………..………………………..Sample

Strut, Tire, Brake………..…..………………………Check

###### NOSE

Windshield………………………………………………Clean

Engine Oil (5min-7max)……………………………Check

Fuel Sump…………………………………..…………..Drain

Prop and Spinner………………………..…………..Check

Alternator Belt…………………………….…………..Check

Air Filter………………………………………………….Check

Nose Wheel and Strut………………………………Check

Static Port………………………………………………Check

#### LEFT WING

##### Fuel Level………….……………………….…………..Check

##### Leading Edge………..………………………………..Check

Pitot Tube……………………………………………….Check

Fuel Vent………………………………………………..Check

Stall Warning Port, Fuel Valve……….………….Check

Tie Down………………..…………………………..Remove

Wing Tip, Aileron, Flap…………………………….Check

Fuel Sump…………………..………………………..Sample

Strut, Tire, Brake…………………………………… Check

### STARTING ENGINE

Preflight…………………………………………….Complete

Weight and Balance/Performance……………..Check

Seats and Belts……………….…………………………..On

Brakes………………………………………………………..Set

Fuel Valve…………………………………………………Both

Circuit Breakers…………………………………………….In

Mixture……………………………………………………..Rich

Throttle………………………………….………….1/4 Open

Carb Heat……………………………………………Off/Cold

Beacon……………………………………………………….On

Radio Master……………………………………………….Off

Prime…………………………………………….As Required

Prop Area…………………………………………………Clear

Master Switch……………………………………………..On

Ignition…………………………………………………….Start

Oil Pressure…………………………In Green In 30 Sec.

Mixture………………………………………..Lean For Taxi

Flaps…………………………………………………………..Up

Radio Master……………………………………………….On

Transponder…………………………………………….STBY

TAXI

ATIS…………………………………………………….Receive

Taxi Clearance………………………….………….Request

Lights…………………………………………….As Required

Taxi Area………………………………………………...Clear

Brakes……………………………………..………….Release

Brakes and Steering…………………………………Check

ENGINE RUN-UP

Brakes………………………………………………………..Set

Doors and Windows…………………………………..Shut

Flight Controls..……………………………………….Check

Trim……………………………………..Check For Takeoff

Fuel Selector.……………………………….…………..Both

Mixture……………………………………………………..Rich

Throttle………………………………..…………..1700 RPM

Suction……………………………………….4.6-5.4 In. Hg

Engine Gauges…………………………………….In Green

Magnetos……………………………………………….Check

(175RPM Max Drop, 50 RPM’s Max Diff.)

Ammeter………………………………………………Positive

Carb Heat……………………………..……………..On/Hot

Throttle……………………………………………………..Idle

Carb Heat……………………..…………………….Off/Cold

Throttle…………………………………………….1000 RPM

Altimeter…………………………………………………….Set

Attitude Indicator…………………………….………….Set

Heading Indicator………………………………………..Set

TAKEOFF

Fuel Selector…………………………….……………..Both

Flaps………………………………………………………….Set

Mixture……………………………………………………..Rich

Carb Heat……………………………………………Off/Cold

Strobes…………………………………………..…………..On

Nav and Landing Lights……………………As Required

Transponder…………………………………….Set To ALT

CLIMB

Flaps…………………………………………………………..Up

Airspeed………………………………………….70-80 KIAS

CRUISE

Throttle……………………………………………………2200

Mixture……………………………………….…………...Lean

Heading Indicator……………………………………Check

Landing Light………………………………………………Off

COMMON FREQUENCIES

LZU………………………………………………………124.10

LZU ATIS……………………………………………….132.27

LZU Ground……………………………………………121.80

LZU ILS RWY 25…………………………………….111.75

WDR CTAF…………………………………………….123.00

WDR ASOS…….……………………………………..118.57

AHN Tower……………………………………………126.30

AHN VOR……..(274 to LZU).……………………109.60

GVL and 19A……….…………………………………122.80

GVL ASOS..……………………………………………126.47

PDK Tower…………………………………………….121.60

PDK ATIS………………………………………………128.40

DESCENT

Fuel Selector.…………………………………………...Both

Mixture…………………………………………………..Adjust

Power…………………………….……………….As Desired

Carb Heat………………………………...On As Required

BEFORE LANDING

Seat, Seat Belts, Shoulder Harness…………..Secure

Fuel Selector.…………………………………………...Both

Mixture……………………………………………………..Rich

Carb Heat……………………………………….…………..On

Landing Light………………………………………………On

NORMAL LANDING

Air Speed………………………..65-75 KIAS (Flaps Up)

Wing Flaps…………………………..…………..As Desired

(0-10 Below 110 KIAS)

(10-30 Below 85 KIAS)

Airspeed……………………..60-70 KIAS (Flaps Down)

SHORT FIELD LANDING

Airspeed……………..………….65-75 KIAS (Flaps Up)

Wing Flaps……………………….……………….Full Down

Airspeed…(Short Final)…………………………60 KIAS

Power……….…………….Idle After Clearing Obstacle

Brakes…………………………………………Apply Heavily

Wing Flaps…………………………….……………..Retract

GO AROUND

Throttle……………………………………………………..Full

Carb Heat……………………………………………Off/Cold

Flaps………………………………….………….Raise To 20

Airspeed……………………….…………………60-65 KIAS

Flaps….(At Safe Alt. And 60 KIAS)……Raise Slowly

In Case Of An Emergency, Contact Advanced Aviation, Inc. at

770-682-9007

AFTER LANDING

Carb Heat…………………….……………………..Off/Cold

Flaps…………………………………………………………..Up

Mixture………………………………………..Lean For Taxi

Transponder…………………………………………….STBY

Landing Light……………………...Off/On As Required

SHUTDOWN

Avionics Master…………………………………………...Off

Nav and Landing Lights…………………….………….Off

Electrical Equipment…………………………………….Off

Magneto Ground Check…………………………...Check

Mixture……………………………………………...Full Lean

Ignition……………………………………………………….Off

Master Switch……………………………………………..Off

Control Lock………………………………………...In Place

Sunscreen……………………………………………...Install

Pitot Tube Cover………………………………………….On

Hobbs/Tach…………………………..Check and Record

Seatbelts………………………………….Fasten On Seats

Belongings and Trash…………………………...Remove

Tie Downs……………………………………………..Secure

Keys, Fuel Cup, Checklist, and Clipboard…..Return

V-SPEEDS

Vr	      (Rotation Speed)	            55 KIAS

Vx	      (Best Angle)	            60 KIAS

Vy	      (Best Rate) 		            73 KIAS

Va	      (Manevering Speed)        82-99 KIAS

Vs	      (Stall Speed)		  44 KIAS

Vso	      (Stall- Full Flaps)	             33 KIAS

Vfe	      (Max Flaps Extended)           85 KIAS

Vno	      (Max Normal Operating)      127 KIAS

Vne	      (Never Exceed)	           158 KIAS

Vglide	      (Best Glide)                         65 KIAS

Max Crosswind                                      15 KIAS

Approach Speeds:

Normal                      65 KIAS

Short Field                  60 KIAS

*EMERGENCY PROCEDURES*

ENGINE FAILURE DURING FLIGHT

Establish Glide Speed……………..…………….65 KIAS

LOOK FOR A FIELD!

Fuel Shutoff Valve………………………………………..On

Fuel Selector…………………………………………….Both

Primer………………………………………..In and Locked

Mixture……………………………………………………..Rich

Ignition…………………………………BOTH (Or START)

IF ENGINE WILL NOT START!

Mayday……………………………………..Transmit 121.5

Transponder………………………………..Squawk 7700

Mixture…………………………………………..Idle Cut-Off

Fuel Shutoff Valve……………………………………….Off

Ignition……………………………………………………….Off

Flaps……………………………………………..As Required Master Switch……………………………………………..Off

Cabin Doors…………………….Unlatch/ Slightly Open

Touchdown………………………………………...Tail Low

CARBURETOR ICING

Throttle………………………………………………………Full

Carb Heat……………………………………………..On/Hot

Mixture…………………………………………………..Adjust

ENGINE RUNNING ROUGH

Carb Heat……………………………………………..…….On

Individual Mags……………………………………….Check

Mixture……………………………………………………..Rich

Fuel Levels……………………………………………..Check

Primer…………………………………………………..Locked

ENGINE FIRE DURING FLIGHT

Mixture………………………………………….Idle Cutt-Off

Fuel Shut-Off Valve……………………………..Off (Out)

Master Switch……………………………………………..Off

Cabin Heat/Air…………………………………………….Off

Airspeed…(To Extinguish Flame)………….100 KIAS